February 4, 2013

Opel ADAM Rally Cup: Technical Data

Engine:	1.6-l ECOTEC 16V, variable camshaft phasing, 
BOSCH Motorsport engine control module

Power output:	185 hp (approx.) at 7,750 rpm

Torque:			190 Nm (approx.) at 6,200 rpm

Transmission:	Sequential Sadev five-speed gearbox, limited-slip differential

Chassis:	REIGER racing suspension, 3-way adjustable, McPherson suspension struts with Uniball mounts, asphalt and gravel specification

Steering:	Electro-hydraulic power steering

Wheels:	EVO Corse aluminum, 6.5 x 16-inch (asphalt), 6 x 15 (gravel)

Braking system:	Hydraulic ‘fly-off’ handbrake, pressure regulation for rear axle, Brembo 4-piston front brake calipers

Brake discs:	310 mm ventilated (asphalt, front), 278 mm ventilated (gravel, front), 264 mm (asphalt and gravel, rear)

Bodywork:	Lightweight shell, integrated safety structure, under-ride guard for engine and fuel tank;

Interior: SPARCO Competition seats (HANS system-compatible), electric fire extinguisher, dry battery

Exterior: ‘OPC Line’ bodywork components, quick release catches on hood and tailgate, towing loops

Electronics:	Motorsport display, LCD gear and shift indicator, MONIT trip computer, central control panel, map-reading lamp

Dimensions:	Length: 3,698 mm, width: 1,720 mm, (adjustable) height: 1,442 mm (asphalt configuration), wheelbase: 2,310 mm, weight: 1,030 kg